MAXWELL INDIA, JAIPUR

GUARANTEED TECHNICAL PARTICULARS

AC STATIC SINGLE PHASE 2 WIRE WHOLE CURRENT ISI MARKED ENERGY METERS WITH LCD DISPLAY

| SL. | PARTICULARS | GUARANTEED COMPLIANCE |
| --- | --- | --- |
| 1 | Name of manufacturer | Maxwell India, Jaipur |
| 2 | Brand Name, Type & Model | “Maxwell”, Type: RS-16 |
| 3 | Standard applicable | IS:13779 with latest amendments |
| 4 | Rating | 5-30A |
| 5 | Accuracy Class | Class–1 |
| 6 | Rated Voltage | 240 V |
| 7 | Basic Current (Ib) | 5A |
| 8 | Imax % of (Ib) | 30A (600% of Ib) |
| 9 | Rated Frequency | 50Hz |
| 10 | Power factor | 0-lag-unity-lead-0 |
| 11 | Starting current | 0.4%Ib (20mA) |
| 12 | Display |  |
|  | Type of Register | Back-lit LCD type |
|  | No. of Digits | 6 complete digits and 2 decimal digits. |
|  | Size of digits | Minimum height 10mm, width 5mm |
|  | Parameters displayed | Cumulative kwh, instantaneous parameters (voltage, current, power-factor, frequency, load-kw) |
| 13 | Type of material used for meter body |  |
|  | Base | Polycarbonate, grey colour |
|  | Meter cover | Polycarbonate, transparent |
|  | Terminal cover | Polycarbonate, grey/transparent |
| 14 | Fixing/ sealing arrangement |  |
|  | Fixing of meter | One fixing provision at top, two fixing holes at bottom. |
|  | Sealing of meter cover to base | Meter sealed with tamper-proof numbered hologram sticker seals on both sides. Additional provision for seals on cover fixing screws. |
|  | Sealing of terminal cover | One no. sealing provision provided on terminal cover fixing screw. |
| 15 | Immunity to external influencing factors and tamper conditions | The meter shall be immune to AC and DC magnetic fields and other external factors in respect of parameters covered in IS 13779 with latest amendments. Tamper proof. |
| 16 | Battery back-up for power off readability | Meter shall have internal battery back-up for power off readability |